QILIN PECTIN A4  PECTIN FOR CONFECTIONERY,JELLIES

TDS

## Description

QILIN PECTIN A4   is   an   extra   slow   setting   purified   high methoxyl  pectin,  extracted  from  apple  peel  and standardized       with       sucrose       and       buffer salt(E331,E452,E330).

| ## Specification Grade confectionary: pH (1% solution): Buffer Capacity: Degree of Esterification: ## Application Confectionery, jellies, pH range: Soluble solids: | 160。± 7 4.0 – 5.0 0.5 – 0.8ml 58 – 62% 3.0 – 3.5 > 75 % |
| --- | --- |

## Usage Levels (Guidelines)

Confectionery, jellies:               1.8 – 2.8 %

We recommend dissolving Pectin in water before addition to the final system.

Sensory characteristics

Appearance:    Free flowing powder.

Colour:              Pale brown powder.

Odour:              Slight, free from off-notes.

Taste:               Slight, free from off-flavours.

## Microbiological characteristics

Total plate count:                        < 1000 cfu/g

Yeast and mould:                       < 100 cfu/g

Coliforms:                                   absent in 1 g

E. coli:                                         absent in 1 g

Staphylococcus aureus:            absent in 1 g

Salmonella:                                absent in 25 g

Chemical characteristics

Pectin content:              > 60 %

Galacturonic acid:         > 65 %

Loss on drying:              < 12 % Ash ,total:                       < 5 %   Acid-insoluble   ash:   <   1   %

Nitrogen content: < 1 %

SO2:*                              < 50ppm Free methyl, ethyl

and isopropyl alcohol:     < 1 %

Heavy metals as lead:     < 15ppm

Lead:                                < 5ppm

Arsenic:                            < 3ppm

Particle size:                    1% retained on 300um * Not used in the process.

## Legal requirements

This product complies with all criteria laid down by ECC/EU, FAO/WHO (JECFA) and FDA/FCC.

## Nutritional information depends on

the standardisation

Energy (per100g) :                      typically 550 - 650kJ

Protein:                                        typically < 0.5%

Carbohydrates :                15 – 25%

Of which sugars:                         15 – 25%

Fat:                                               < 0.5%

Fibre:                                           typically 75 – 85%

## Packaging & Storage

Packaging:        Paper with polythene liner  (25kg

net)

Storage:                         Cool and dry

Shelf life:                        24 months

## Risks and Handling

A Material Safety Data Sheet is available on request.

The customer must take sole responsibility in the use and control of this product according to their countries current legislations.