QILIN PECTIN A7 FOR JAMS,JELLIES

TDS

# Description

QILIN   pectin   type   A7  is   a    purified   low

methoxyl  pectin,  extracted from  apple  pomace and standardised with sugar (sucrose) .

Composition

| Pectin | E 440 |
| --- | --- |
| Sucrose |  |

# Typical application

Intended for  use  as a gelling  agent  in systems with soluble solids of < 50%

Usage levels (Guidelines)

Jams, jellies      0.5 -  1.7%

The  optimal  dosage  depends  on  pH,  soluble  solids and calcium content  in the application.

We   recommend   dissolving   pectin   in  water    before addition to the final system.

Sensory characteristics

| Item | Index |
| --- | --- |
| Appearance | free flowing powder |
| Colour | pale brown powder |
| Odour | slight, free from off-notes |
| Taste | slight, free from off-flavours |

Specification

| Item | Index |
| --- | --- |
| pH (2% solution) | 3.0 - 3.8 |
| Degree of esterification | typically31 - 36 % |
| Grade with calcium | 100 ±5 |
| Galacturonic acid | ≥ 65 % |
| Loss on drying | ≤ 12 % |
| Acid-insoluble ash | ≤ 1 % |

Chemical characteristics

| Item | Index |
| --- | --- |
| Nitrogen content: | ≤ 1 % |

| SO2* | ≤ 10 mg/kg |
| --- | --- |
| Free methyl, ethyl and isopropyl alcohol | ≤ 1 % |
| Lead | ≤ 2 mg/kg |
| Arsenic | ≤ 3 mg/kg |
| Mercury | ≤ 1 mg/kg |
| Cadmium | ≤ 1 mg/kg |

* Not used in the process.

Microbiological characteristics

| Item | Index |
| --- | --- |
| Total plate count | ≤ 500 CFU/g |
| Yeast and mould | ≤ 100 CFU/g |
| Coliforms | < 3.0 MPN/g |
| E. coli | < 3.0 MPN/g |
| Staphylococcus aureus | absent in 25 g |
| Salmonella | absent in 25 g |

# Legal requirements

This product complies with all criteria  laid down by EU,   FAO/WHO   (JECFA)  and    FDA/ FCC  and    is GMO and Allergen free according to  EC directives.

# Quality assurance system

ISO 9001 ,  ISO 22000 ,  BRC,  Halal, Kosher.

Packaging & storage

| Packaging | paper with polythene liner ( 25 kg net ) |
| --- | --- |
| Storage | cool and dry |
| Shelf life | 24 months |

# Risks and handling

A  Material Safety Data Sheet is available on request.

。